USDOT GlidePath Project

System Acceptance Test Report

Version 2.1  November 10, 2015

LEIDOS

Security Classification: Unclassified

©Leidos, 2015. All rights reserved.

Table of Contents

Table of Contents	2

1.0 Test Results Summary Report	3

2.0 Test Cases	4

2.1 Security	4

2.2 Infrastructure	6

2.3 Vehicle	8

2.4 Algorithm	18

2.5 Driver-Vehicle Interface	26

3.0 Requirements – Test Case Traceability Matrix	29

Appendix A - GlidePath Security Protocol Agreement	32

Appendix B - Acronyms	34

# 1.0 Test Results Summary Report

| Agenda # | Test Case ID | Test Case Name | Result |
| --- | --- | --- | --- |
|  | Security |  |  |
| 1 | TO17_SEC_01 | Prototype Security |  |
| 4 | TO17_SEC_02 | PC Password Protection |  |
|  | Infrastructure |  |  |
| 6 | TO17_INF_01 | Infrastructure Readiness |  |
|  | Vehicle |  |  |
| 2 | TO17_VEH_01 | Documentation of Vehicle Functionality |  |
| 3 | TO17_VEH_02 | In-Vehicle Equipment |  |
| 5 | TO17_VEH_03 | Basic Longitudinal Control |  |
| 7 | TO17_VEH_04 | Vehicle Behavior |  |
|  | Algorithm |  |  |
| 8 | TO17_ALG_01 | Demonstration of Algorithm Functionality |  |
| 10 | TO17_ALG_02 | Demonstration of Algorithm Process |  |
|  | Data |  |  |
| 11 | TO17_DATA_01 | Output Data Post-Processing |  |
|  | Driver-Vehicle Interface |  |  |
| 9 | TO17_DVI_01 | Driver-Vehicle Interface |  |

# 2.0 Test Cases

The following tables include the specific procedures that will verify the functionality of the prototype in accordance with the technical requirements. As each procedure is implemented, the test results (outcome, pass/fail) will be noted to the right and will be accompanied by the FHWA General Task Manager’s initials where each technical requirement is verified.

Advisory: The test procedures included below are written for Authorized Users who have training in the GlidePath Operating Procedures. Please review the GlidePath Vehicle Basic Manual and read through the full test case procedures before commencement of each test to ensure all safety protocols are followed.

For test cases that denote required Data Outputs, relevant documentation will be attached to the signed copy of the test report and stored on file for reference.

## 2.1 Security

| Test Case # | TO17_SEC_01 |
| --- | --- |
| Test Case | Prototype Security |
| Reference |  |
| Objective | To document the agreement to comply with the requirements listed below, including physical and operational security. |
| Requirements verified | • TO17_SEC_01v1 – Physical Security • TO17_SEC_02v1 – Operational Security |
| Brief Description | The team will provide signed agreements to abide by and enforce the listed security protocols. |
| Data Outputs | Signed agreements will be provided to the FHWA General Task Manager and saved on file. |

The signature below attests to receipt of signed Security Protocol Agreements from the

GlidePath staff:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

| Test Case # | TO17_SEC_02 |
| --- | --- |
| Test Case | PC Password Protection |
| Reference |  |
| Objective | Verification of the restrictions implemented for system access to the in-vehicle processors. |
| Requirements verified | • TO17_SEC_03v1 – System Access |
| Brief Description | The team will log into the in-vehicle PCs using the documented user name and password which will be shared with the FHWA General Task Manager and included in the Prototype Operation and Maintenance document. |
| Data Outputs | none |

EVALUATION PROCEDURES

| Step | Instruction | Expected Outcome / Requirement Verified | Actual Result / Pass or Fail | Initial |
| --- | --- | --- | --- | --- |
| 1 | With the vehicle parked in the garage, plug into the workstation monitor. |  |  |  |
| 2 | Turn on the workstation monitor and type in the username and password (to be provided by test conductor) | TO17_SEC_ 03v1 – System Access |  |  |

The signature below indicates acceptance of the Prototype as compliant with the requirements listed in this test case:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

## 2.2 Infrastructure

The infrastructure requirements will be verified by visual inspection and demonstration prior to any field testing. The Intelligent Intersection at TFHRC will be configured to support the application using a fixed

time signal cycle that is broadcast over DSRC.

| Test Case # | TO17_INF_01 |
| --- | --- |
| Test Case | Infrastructure Readiness |
| Reference |  |
| Objective | Verification of the fixed-time signal cycle and the accurate broadcast of SPaT, MAP, and WSAs over DSRC. |
| References | • |
| Requirements verified | • TO17_FUN_17v1 – Signal Control • TO17_FUN_18v1 – DSRC Communication (Infrastructure) |
| Brief Description | Using the appropriate tools and methods, the test conductor will verify that the signal cycle of the intersection is set to fixed-time and will not be actuated by any of the local loop detectors. Additionally, the test conductor will verify that SPaT, MAP, and WSAs are being broadcast over the air using DSRC. |
| Entrance Criteria | • Vehicle is equipped with a DSRC Protocol Analyzer |
| Configuration | • Traffic controller is set to normal function (not flash) • SPaT Black Box is connected to traffic controller over Ethernet and pointed to the test RSU |
| Exit Criteria | • Loop detectors are not active • Traffic signal operates with constant, fixed time cycle • SPaT, MAP, and WSAs are captured from the vehicle |
| Data Outputs | Saved capture from DSRC Protocol Analyzer and documentation of observed signal timing for at least three cycles. |

EVALUATION PROCEDURES

The following instructions will guide the relevant test staff through the necessary protocols to verify that the traffic signal operates with fixed timing

| Step Instruction Expected Actual Initial Outcome / Result / Requirement Pass or Verified Fail |  |  |  |
| --- | --- | --- | --- |
| 1 At the Intelligent Intersection’s traffic controller cabinet, verify that all loop detectors are turned off and that the controller is set to fixed time operation in accordance with the timing protocol listed below. |  |  |  |
| 2 Data Recorder shall compose a record of the observed signal timing from the Econolite Traffic Signal Controller | TO17_FUN_ |  |  |
|  | 17v1 |  |  |

| Step Instruction Expected Actual Initial Outcome / Result / Requirement Pass or Verified Fail |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | Lead Passenger shall run DSRC Protocol Analyzer to identify relevant message sets and save a capture file with the following name: YearMonthDay_TOINF01_Pass/Fail_UTCTim e.pcap where “YearMonthDay” and “UTCTime” reflect the capture start time. | SPaT, MAP, and WSAs are observed MSG/PSID SPAT/BFE0 MAP/BFF0 WSA/35 |  |  |  |
| 4 Post Processing Saved .pcap files should be analyzed to verify the frequency of the broadcasts meet the requirements |  |  | TO17_FUN_ |  |  |
|  |  |  | 18v1 |  |  |

|  | Expected Phase Length |  |  | Actual Phase Length |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Phase | G | Y | R | G | Y | R |
| 2/6 (NB/SB) | 27 | 3 | 30 |  |  |  |
| 4/8 (EB/WB) | 27 | 3 | 20 |  |  |  |

The signature below indicates acceptance of the Prototype as compliant with the requirements listed in this test case:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

## 2.3 Vehicle

The vehicle related requirements require multiple test cases be run to verify the component functionalities of the prototype that will ultimately build to the full application. Many of these test cases reference the

“XGV Start Up Procedures” in their Entrance Criteria; these procedures are documented as part of the

XGV training materials and a quick-start guide will be mounted on the passenger side of the vehicle.

| Test Case # | TO17_VEH_01 |
| --- | --- |
| Test Case | Documentation of Vehicle Functionality |
| Reference |  |
| Objective | Verify documentation of vehicle settings and configurations that will not be field tested, such as automated lateral control and the maximum acceleration thresholds. |
| Requirements verified | • TO17_FUN_09v1 – Acceleration Control Commands • TO17_FUN_10v1 – Deceleration Control Commands • TO17_INT_02v1 – Standard Command Structure |
| Brief Description | Review of documentation that prototype vehicle has the capability for automated lateral control and the maximum speed and acceleration thresholds for automated longitudinal control of the vehicle. Verification that vehicle utilizes JAUS via Ethernet as the command structure for control commands to the vehicle. |
| Configuration |  |
| Data Outputs | User manual for XGV |

RELEVANT DOCUMENTATION

The following requirements have been included for quick reference and are supported by documentation from the TORC ByWire XGV User Manual Hybrid Escape Drive-by-Wire Platform, version 1.5.

•	TO17_FUN_09v1 – Acceleration Control Commands The prototype SHALL have internal ‘closed-loop control’ to accelerate to a target speed by commanding the target speed and acceleration.

o	The TORC ByWire XGV is equipped with closed loop drive-by-wire control that allows a user to command a desired acceleration and speed. This functionality is described in Section

5.1 “Control Schema Overview” on page 8 of the User Manual.

•	TO17_FUN_10v1 – Deceleration Control Commands The prototype SHALL have internal ‘closed-loop control’ to decelerate to a target speed by commanding the target speed and deceleration, within the capabilities of the vehicle.

o	The TORC ByWire XGV is equipped with closed loop drive-by-wire control that allows a user to command a desired acceleration and speed. This functionality is described in Section

5.1 “Control Schema Overview” on page 8 of the User Manual.

•	TO17_INT_02v1 – Standard Command Structure

The prototype SHALL utilize JAUS via Ethernet (SAE/TP 2009-01-3250) as the command structure to enable control commands to be sent to control the vehicle.

The signature below indicates acceptance of the Prototype as compliant with the requirements listed in this test case:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

| Test Case # | TO17_VEH_02 |
| --- | --- |
| Test Case | In-Vehicle Equipment |
| Reference |  |
| Objective | Visually inspect DSRC Communication, Positioning, and Processing components installed in the vehicle, as well as Ethernet cable connections. Component functionality is documented in component user manuals which will be available for reference. |
| Requirements verified | • TO17_FUN_14v1 – DSRC Communication • TO17_FUN_15v1 – Positioning • TO17_FUN_19v1 – In-Vehicle Processing • TO17_INT_01v1 – Ethernet Interface |
| Brief Description | The project team will provide an in-person overview of the vehicle components and interfaces as compliant with the requirements listed above. |
| Configuration |  |
| Data Outputs |  |

RELEVANT DOCUMENTATION

This test case will be completed by visual inspection of the in-vehicle equipment components, including the DSRC After-market Safety Device (ASD), Positioning System, In-Vehicle Processing capability, and Ethernet interface, as pictured below, as well as the CANBus interface, which is not visible in this photo.

Positioning System	DSRC Communication

Ethernet Interface

In-Vehicle

Processing

The signature below indicates acceptance of the Prototype as compliant with the requirements listed in this test case:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

| Test Case # | TO17_VEH_03 |
| --- | --- |
| Test Case | Basic Longitudinal Control |
| Reference |  |
| Objective | Verify capability of the vehicle to receive and implement electronic speed commands in the forward and reverse direction. |
| Requirements verified | • TO17_FUN_01v1 – Automated Longitudinal Control • TO17_FUN_02v1 – Directional Control • TO17_FUN_03v1 – Automated Lateral Control • TO17_FUN_04v1 – Decoupled Longitudinal and Lateral Control |
| Brief Description | Using the TORC-furnished XGV Operator Control Unit (OCU) application, the test conductor will demonstrate the basic capabilities of the vehicle to operate under automated longitudinal control in compliance with the requirements. |
| Configuration | In accordance with the XGV Start Up Procedures, the vehicle must be in the Automated state for this test |
| Data Outputs | Saved capture from DSRC Protocol Analyzer and documentation of observed signal timing for at least three cycles. |

EVALUATION PROCEDURES

Step Instruction Expected Outcome / Actual Result / Initial Requirement Verified Pass or Fail

1 Start-up the vehicle in “Manual” mode and pull out of the garage, and set the car to “Automated” mode and enable Manual Steering through the dashboard display.

2 Connect PC to Ford network switch via Ethernet, configure static IP with 192.168.0.XX

3 Plug the X-Box controller from the center console into the PC’s USB port

4 Start Up TORC ByWire XGV OCU Software

| Step Instruction Expected Outcome / Actual Result / Initial Requirement Verified Pass or Fail |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 Verify communication is Red “Communications established with XGV Time Out” error will be present if communication is not established |  |  |  |  |  |  |  |  |  |
| 6 Select “Request MPD” Control |  |  |  |  |  |  |  |  |  |
| 7 Click on Motion Profile Driver Tab, and use X- Box Controller to move the vehicle. The right toggle is used for longitudinal control; press the toggle forward to accelerate forward and pull the toggle back to accelerate in reverse. |  | TO17_FUN_01v1 |  |  |  |  |  |  |  |
|  |  | – Automated Longitudinal |  |  |  |  |  |  |  |
|  |  | Control |  |  |  |  |  |  |  |
|  |  | TO17_FUN_02v1 |  |  |  |  |  |  |  |
|  |  | – Directional |  |  |  |  |  |  |  |
|  |  | Control |  |  |  |  |  |  |  |
| 8 Using the dashboard controller, enable automated steering and disengage automated speed control. |  |  |  |  |  |  |  |  |  |
| 9 While the driver controls the gas and brake pedals, have the passenger use the X-Box Controller’s left toggle to move forward and backward for steering control. |  | TO17_FUN_03v1 |  |  |  |  |  |  |  |
|  |  | – Automated |  |  |  |  |  |  |  |
|  |  | Lateral Control |  |  |  |  |  |  |  |
|  |  | TO17_FUN_04v1 |  |  |  |  |  |  |  |
|  |  | – Decoupled |  |  |  |  |  |  |  |
|  |  | Longitudinal and |  |  |  |  |  |  |  |
|  |  | Lateral Control |  |  |  |  |  |  |  |

The signature below indicates acceptance of the Prototype as compliant with the requirements listed in this test case:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

| Test Case # | TO17_VEH_04 |
| --- | --- |
| Test Case | Vehicle Behavior |
| Reference |  |
| Objective | Verify that the vehicle behavior is compliant with the requirements listed below and activated using the required control mechanisms. |
| Requirements verified | • TO17_FUN_05v1 – Automated Control Switch • TO17_FUN_06v1 – Manual Override • TO17_FUN_07v1 – Additional Control Mechanism • TO17_BEH_01v1 – Ignition Off to Park • TO17_BEH_02v1 – Park to Manual • TO17_BEH_03v1 – Park to Automated • TO17_BEH_04v1 – Automated to Manual • TO17_BEH_05v1 – Manual to Automated • TO17_BEH_06v1 – Automated to EcoDrive • TO17_BEH_07v1 – EcoDrive to Automated • TO17_BEH_08v1 – EcoDrive to Idle • TO17_BEH_09v1 – Idle to Automated • TO17_BEH_10v1 – Idle to Manual |
| Brief Description | The test conductor will demonstrate the transition between each state listed in the requirements to verify that the vehicle behaves as expected, especially as dictated by the required physical control mechanisms (activation key, brake pedal, emergency override button). |
| Entrance Criteria | The vehicle must be: 1 - Disconnected from shore power ; and, - Parked outside at TFHRC. The Test Driver must be seated in the driver’s seat. The Driver and all Passengers must have their safety belts fastened. Traffic control staff must be deployed and ready to monitor and manage traffic for the test runs. |
| Configuration | - Activation Key must be turned to “0”; |
| Data Outputs |  |

1 For more information about these settings, please review the GlidePath Vehicle Basic Manual.

EVALUATION PROCEDURES

The following procedures are designed to demonstrate how the vehicle transitions between different states. For quick reference, the State Diagram is included below the procedures table.

ADVISORY: Before engaging automated control of the vehicle, the driver must enable “Manual Steering” through the XGV Radio

Interface. Please see the “XGV Start-Up Procedures” for more information.

| Step Instruction Expected Outcome / Requirement Actual Result / Verified Pass or Fail |  |  |
| --- | --- | --- |
| 1 Turn the ignition key to start the vehicle. Driver dash will display “Ready to Drive.” |  |  |
| 2 With the vehicle running and the gear shift in P, turn XGV will turn on and engage the the activation key from “0” to “1.” brake. TO17_BEH_01v1 – Ignition Off to Park TO17_FUN_05v1 – Automated Control Switch |  |  |
| 3 In the center console, override the TORC link to the Pause and Stop lights on the XGV handheld SafeStop controller, by pressing the Bypass SafeStop controller will turn green, the button three times, waiting in between for the blue Link light will remain red, and the light to blink. brake will release. |  |  |
| 4 Move the gear shift from “P” to “D” and drive the | TO17_BEH_02v1 – Park to Manual |  |
| vehicle in manual mode to the Experimental Start Location. |  |  |

Step Instruction Expected Outcome / Requirement Actual Result / Verified Pass or Fail

5 With your foot on the brake, move the gear shift from “D” to “N,” depress the Yellow Emergency Manual Override button.

6 Release the brake and Yellow Emergency Manual The XGV will activate and engage the Override button. brake. TO17_BEH_05v1 – Manual to Automated

Caution: Before engaging automated control, use the XGV Radio Interface to enable “Manual Steering.” For additional information, please reference the XGV Start-Up Procedures document and the XGV User Manual.

7 Turn on the tablet display using the power button, Screen will display a black and red swipe right to unlock the screen and select the menu screen. “GlidePath” App Icon.

8 Click “START Glidepath” and then select “OK.” Screen will display a pop-up that reads “Done starting Glidepath!” that will disappear upon clicking “OK.”

9 Click “Navigate to Glidepath DVI.” Screen will display the System Advisory.

10 Read the System Advisory and select “OK” to Screen will display Driver Input screen proceed with the test case. with operational speed selection and current signal phase and timing information.

11 Select “20” for the Operational Speed using the down arrow.

12 Verify that all elements of Activation Sequence are Green and that Signal Status is actively counting SPaT

| Step Instruction Expected Outcome / Requirement Actual Result / Verified Pass or Fail |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 To start automated speed control, wait for the SPaT to The vehicle siren will sound and the read 0 seconds of Red time and prepare to assume light will flash as the GlidePath manual steering and select “Go.” application engages automated speed control and accelerates forward. TO17_BEH_06v1 – Automated to EcoDrive |  |  |  |  |  |  |  |  |  |  |
| 14 When there are 5 seconds of red remaining, a “GO” button will flash green on the screen. Visually inspect the intersection and surrounding area for safety conflicts and press “GO” on the DVI display when ready to proceed. NOTE: The application will not resume until BOTH the light turns green and the driver has selected GO. |  | The vehicle will come to a complete stop at the red light and EcoDrive will halt until the driver re-engages the DVI. |  |  |  |  |  |  |  |  |
|  |  | TO17_BEH_07v1 – EcoDrive to |  |  |  |  |  |  |  |  |
|  |  | Automated |  |  |  |  |  |  |  |  |
| 15 After the vehicle passes through the intersection and |  | TO17_BEH_08v1 – EcoDrive to Idle |  |  |  |  |  |  |  |  |
| approaches the Experimental End Location, the application will automatically turn off EcoDrive and prompt the driver to resume Manual control. |  |  |  |  |  |  |  |  |  |  |
| 16 When the DVI displays the following message: “Experiment ended; driver resume manual control,” tap the brake, shift to D and resume manual control of the vehicle. |  | When the brake is depressed, automated control will disengage and the vehicle will be in the “IDLE” state. When the gear shift moves from “N” to “D” the vehicle will return to the “MANUAL” state. |  |  |  |  |  |  |  |  |
|  |  | TO17_FUN_07v1 – Additional |  |  |  |  |  |  |  |  |
|  |  | Control Mechanism |  |  |  |  |  |  |  |  |
|  |  | TO17_BEH_10v1 – Idle to Manual |  |  |  |  |  |  |  |  |

Step	Instruction	Expected Outcome / Requirement

Verified

Actual Result / Pass or Fail

17	At the end of the experimental run, turn the vehicle around and return to the Experimental Start Location.

The following steps demonstrate the functionality of additional control mechanisms, including the emergency override button, gear shift, and brake pedal.

18	Move the gearshift to P to park the vehicle.

19	Release the brake pedal, reset the yellow emergency override button and move the gearshift to N.

TO17_BEH_03v1 – Park to

Automated

20	Move the gearshift to D and assume manual control of the vehicle to move forward and backward.

TO17_BEH_04v1 – Automated to

Manual

21	Move the gearshift back to N, release the brake and reset the yellow emergency button

22	Engage the application using the DVI, and press

“GO” when ready.

23	As the vehicle begins to accelerate, tap the brake to manually override the application and transition to the “Idle” state.

24	Release the brake, and press and release the yellow emergency override button to return to the “Automated” state and re-engage the application through the DVI.

TO17_FUN_06v1 – Manual Override

TO17_BEH_09v1 – Idle to

Automated

The signature below indicates acceptance of the Prototype as compliant with the each of the requirements listed in this test case and behaves in accordance with the state diagram depicted above:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

## 2.4 Algorithm

The primary functionality of the algorithm will be verified through both a high-level, and ‘behind-the- scenes’ demonstration, as well as a post-processing analysis of the output data.

| Test Case # | TO17_ALG_01 |
| --- | --- |
| Test Case | Demonstration of Algorithm Functionality |
| Reference |  |
| Objective | Verify that application functionality is consistent with the behavior described in the relevant requirements. |
| Requirements verified | • TO17_FUN_20v1 – Vehicle Trajectory Planning Algorithm |
| Brief Description | The project team will conduct a preliminary drive in the vehicle from the test start point through the intersection to completion with the FHWA General Task Manager to demonstrate the operational application for the scenario in which the vehicle comes to a full stop at a red light and must be re-engaged by the driver. This drive will be repeated for the remaining three (3) scenarios to demonstrate the full-functionality of the algorithm at different phases of the signal cycle. |
| Entrance Criteria | The Test Driver must be seated in the driver’s seat. The Driver and all Passengers must have their safety belts fastened. Traffic control staff must be deployed and ready to monitor and manage traffic for the test runs. |
| Configuration |  |
| Data Outputs |  |

EVALUATION PROCEDURES

Step Instruction Expected Outcome / Requirement Actual Result / Verified Pass or Fail

1 Turn on the tablet display using the power button, swipe right to unlock the screen and select the “GlidePath” App Icon.

2 Click “START Glidepath” and then select “OK.”

3 Click “Navigate to Glidepath DVI.”

4 Read the System Advisory and select “OK” to proceed with the test case.

5 Select “20” for the Operational Speed using the down arrow.

6 Verify that all elements of Activation Sequence are Green and that Signal Status is actively counting SPaT

7 To start automated speed control, wait for the SPaT to read 0 seconds of Red time and prepare to assume manual steering and select “Go.”

8 When there are 5 seconds of red remaining, a “GO” button will flash green on the screen. Visually inspect the intersection and surrounding area for safety conflicts and press “GO” on the DVI display when ready to proceed. NOTE: The application will not resume until BOTH the light turns green and the driver has selected GO.

| Step Instruction Expected Outcome / Requirement Actual Result / Verified Pass or Fail |  |  |  |
| --- | --- | --- | --- |
| 9 After the vehicle passes through the intersection and approaches the Experimental End Location, the application will automatically turn off EcoDrive and prompt the driver to resume Manual control. |  |  |  |
| 10 When the DVI displays the following message: “Experiment ended; driver resume manual control,” tap the brake, shift to D and resume manual control of the vehicle. |  |  |  |
| 11 At the end of the experimental run, turn the vehicle around and return to the Experimental Start Location. |  |  |  |
| 12 Repeat steps 2-11 for the remaining three scenarios using the start times in the table below. | TO17_FUN_20v1 – Vehicle Trajectory |  |  |
|  | Planning Algorithm |  |  |

Scenario Scenario Signal Phase Signal Number Description Timing

1 Maintain Speed Red 30

2 Speed Up Red 5

3 Full Stop Red 0

4 Slow Down Green 5

The signature below indicates acceptance of the Prototype as compliant with the requirements listed in this test case:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

| Test Case # | TO17_ALG_02 |
| --- | --- |
| Test Case | Demonstration of Algorithm Process |
| Reference |  |
| Objective | Verify that the software components that address the algorithm-specific requirements listed below both exist and function properly. |
| Requirements verified | • TO17_FUN_16v1 – Application • TO17_FUN_27v1 – Map Matching |
| Brief Description | The project team will conduct a drive in the vehicle with the FHWA General Task Manager with a remote connection to the in-vehicle PC to demonstrate the functionality of the individual software components listed above. |
| Entrance Criteria | The Test Driver must be seated in the driver’s seat. The Driver and all Passengers must have their safety belts fastened. Traffic control staff must be deployed and ready to monitor and manage traffic for the test runs. |
| Configuration | • PC is configured to access the in-vehicle processor over 4G |
| Data Outputs |  |

REV 2.0	21	GlidePath Prototype Requirements Compliance Test Plan

Use, duplication, or disclosure is subject to the restrictions as stated on the cover page.

EVALUATION PROCEDURES

| Step Instruction Expected Outcome / Requirement Verified Actual Result / Pass or Fail |  |  |
| --- | --- | --- |
| 1 The Driver shall turn on the vehicle and manually drive to the start location. |  |  |
| 2 Initialize the Glidepath software via the DVI. |  |  |
| 3 Wait until the vehicle has computed a value for distance to This computation is required for Glidepath stop bar. to start autonomous control and requires processed MAP data and GPS data to match the vehicle to a lane. TO17_FUN_27v1 – Map Matching |  |  |
| 4 Start the Glidepath run at one of the experimentally determined scenario start conditions. |  |  |
| 5 The Driver shall control the lateral motion of the vehicle for the duration of the run as well as engage manual override in the event of unsafe operation. |  |  |
| 6 The Data Recorder shall observe the vehicle’s behavior as it approaches the intersection. |  |  |
| 7 Reset conditions and conduct three more runs to demonstrate the remaining three algorithm scenarios. |  |  |
| 8 Post-Process the logs generated during each scenario to see | TO17_FUN_16v1 – Application |  |
| algorithm operation parameters, inputs, and outputs. |  |  |

The signature below indicates acceptance of the Prototype as compliant with the requirements listed in this test case:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

REV 2.0	22	GlidePath Prototype Requirements Compliance Test Plan

Use, duplication, or disclosure is subject to the restrictions as stated on the cover page.

| Test Case # | TO17_DATA_01 |
| --- | --- |
| Test Case | Output Data Post-Processing |
| Reference |  |
| Objective | Evaluate prototype data output logs to ensure all data requirements are met. |
| Requirements verified | • TO17_FUN12v1 – Jerk Threshold • TO17_FUN22v1 – DSRC Message Data • TO17_FUN23v1 – Vehicle Positioning System • TO17_FUN24v1 – Vehicle Longitudinal Control • TO17_FUN25v1 – MAP Parser • TO17_FUN26v1 – SPaT Parser • TO17_DAT_01v1 – Data Logging Elements • TO17_DAT_02v1 – Data Logging Frequency • TO17_DAT_03v1 – Data Logging Time Protocol • TO17_DAT_04v1 – Data Storage |
| Brief Description | The project team will provide an annotated sample of the output data along with an actual output file from one of the test runs to both the evaluation team and FHWA General Task Manager to verify that all related data requirements are fulfilled by the application. |
| Configuration |  |
| Data Outputs | Full log of live demonstrations as well as an annotated sample output log for cross- reference. |

Step	Instruction	Expected Outcome / Requirement Verified

Actual Result / Pass or

Fail

1	Connect to the GlidePath PC

via WinSCP the local IP

192.168.0.4

2	Navigate to /opt/glidepath/logs	TO17_DAT_04v1 – Data

Storage

3	Open relevant .log file

4	Compare with sample output below

5	Review highlighted data elements in the sample data sets on the following pages

TO17_DAT_01v1 – Data

Logging Elements

TO17_FUN_12v1 – Jerk

Threshold

TO17_FUN22v1 – DSRC Message Data

TO17_FUN23v1 – Vehicle

Positioning Data

TO17_FUN24v1 – Vehicle

|  | Longitudinal Control |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TO17_FUN25v1 – MAP |  |  |  |  |  |
|  | Parser |  |  |  |  |  |
| 6 Review data logging frequency in logs to verify information is | TO17_DAT_02v1 – Data |  |  |  |  |  |
|  | Logging Frequency |  |  |  |  |  |
| recorded at least 1 Hz. |  |  |  |  |  |  |

Figure 1. Annotated sample .log file output

TIME_SINCE_FIRST_MOTION	17.348

OPERATING_SPEED	8.9408

SPEED_COMMAND	11.06366

SMOOTHED_SPEED	10.16474

SPEED	10.1618

ACCELERATION	0.245428

JERK	-0.04297

LATITUDE	38.955

LONGITUDE	-77.1478

DIST_TO_STOP_BAR	116.1

SIGNAL_PHASE	GREEN SIGNAL_TIME_TO_NEXT_PHASE		12.9

SIGNAL_TIME_TO_THIRD_PHASE	15.9

MOTION_STATUS	Coast

LANE_ID	12

CYCLE_GPS	1

CYCLE_MAP	31

CYCLE_SPAT	33

CYCLE_XGV	19

CYCLE_EAD	1

CYCLE_XGV_COMMAND	24

XgvStatus{statusCode=READY, manualOverrideEngaged=false, safeStopPauseEngaged=false, safeStopStopEngaged=false, safeStopLinkStatus=1, externalSafeStopEngaged=false, computerSteeringControlEngaged=true, computerSpeedControlEngaged=false,

TO17_FUN12v1

Jerk Threshold

TO17_FUN22v1

DSRC Message

Data

TO17_FUN23v1

Vehicle Positioning

Data

TO17_FUN25v1

MAP Parser

TO17_FUN26v1

SPaT Parser

XGV_STATUS

MAP_MESSAGE SPAT_MESSAGE STATUS_MESSAGE

doorPauseEngaged=false, errorPauseEngaged=false, emergencyManualOverrideEngaged=false, steeringNeedsInitialization=false, steeringInitializationNeedsUserInput=false, on=true, mainFuelOrEnergyOn=true, auxiliaryEnergyOrFuelOn=false, autoStartInProgress=false, autoShutdownInProgress=false, parkingBrakeSet=false, hornEngaged=false, gear=DRIVE}

Figure 2. Sample trajectories from experimental runs – TO17_FUN24v1 – Vehicle Longitudinal Control

The signature below indicates acceptance of the Prototype as compliant with the requirements listed in this test case:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

## 2.5 Driver-Vehicle Interface

The various screens used in the Driver-Vehicle Interface will be printed for off-line review by the FHWA General Task Manager and used during the test drives for comparison.

| Test Case # | TO17_DVI_01 |
| --- | --- |
| Test Case | Driver-Vehicle Interface |
| Reference |  |
| Objective | Verify that all DVI related requirements are met. |
| Requirements verified | • TO17_DVI_01v1 – System Activation Switch • TO17_DVI_02v1 – System Status Indicator • TO17_DVI_03v1 – Intersection Departure Switch • TO17_DVI_04v1 – Signal Phase and Timing Display • TO17_DVI_05v1 – Motion Status Indicator • TO17_DVI_06v1 – Auditory Indicator • TO17_DVI_07v1 – Auditory Alert |
| Brief Description | The project team will provide a printed copy of the various DVI screens for preview by the FHWA General Task Manager to verify the content of the DVI and a specific test drive will be conducted to verify the compliance of the DVI functionality in accordance with the requirements listed above. |
| Entrance Criteria | The Test Driver must be seated in the driver’s seat. The Driver and all Passengers must have their safety belts fastened. Traffic control staff must be deployed and ready to monitor and manage traffic for the test runs. |
| Configuration | The vehicle should be at the Experimental Start Location, in Automated mode. |
| Data Outputs | Printed DVI material with annotations |

EVALUATION PROCEDURES

Step	Instruction	Expected Outcome / Requirement Verified

Actual Result / Pass or

Fail

1	With the vehicle in Automated mode at the Experimental Start Location, select the GlidePath application icon on the tablet display.

2	Select “Start GlidePath” and click “OK” on the pop-up menu that reads “Successfully started GlidePath.”

Black main menu screen will appear

Step Instruction Expected Outcome / Actual Result / Pass or Requirement Verified Fail

3 Select “Navigate to GlidePath The “Authorized Users DVI” and click “OK” after Only” page will display, reading the advisory warning followed by the on the following page. operational speed selection page

4 Review the DVI screen (Screen 2) to ensure it is consistent with the graphic and the relevant requirements listed below.

5 Select the desired operational Application will engage, speed, prepare to control and DVI will change to steering, and select “GO” Screen 3

6 Have a passenger review the DVI screen (Screen 3) to ensure it is consistent with the graphic and the relevant requirements listed below.

Figure 3. From top to bottom: DVI Screens 1-4

The signature below indicates acceptance of the Prototype as compliant with the requirements listed in this test case:

FHWA GTM Signature	Date	GlidePath Project Lead	Date

# 3.0 Requirements – Test Case Traceability Matrix

The following table is a comprehensive list of the requirements from the Final GlidePath Requirements document, version 6.1 dated April 20,

2015. Each line lists the Test Case Number for each requirement that has not been removed.

Table 1. Requirement-Test Case Traceability Matrix

| Task Category ID Description Test Case # Test Case Name |  |
| --- | --- |
| TO17 FUN 1 Automated Longitudinal Control TO17_VEH_03 Basic Longitudinal Control |  |
| TO17 FUN 2 Directional Control TO17_VEH_03 Basic Longitudinal Control |  |
| TO17 FUN 3 Automated Lateral Control TO17_VEH_03 Basic Longitudinal Control |  |
| TO17 FUN 4 Decoupled Longitudinal and Lateral Control TO17_VEH_03 Basic Longitudinal Control |  |
| TO17 FUN 5 Automated Control Switch TO17_VEH_04 Vehicle Behavior |  |
| TO17 FUN 6 Manual Override TO17_VEH_04 Vehicle Behavior |  |
| TO17 FUN 7 Additional Control Mechanism TO17_VEH_04 Vehicle Behavior |  |
| TO17 FUN 8 Emergency Stop Button REMOVED |  |
| TO17 FUN 9 Acceleration Control Commands TO17_VEH_01 Documentation of Vehicle Functionality |  |
| TO17 FUN 10 Deceleration Control Commands TO17_VEH_01 Documentation of Vehicle Functionality |  |
| TO17 FUN 11 Response Time | REMOVED |
| TO17 FUN 12 Jerk Threshold TO17_VEH_04 Vehicle Behavior |  |
| TO17 FUN 13 Stopping Capability | REMOVED |
| TO17 FUN 14 DSRC Communication TO17_VEH_02 In-Vehicle Equipment |  |
| TO17 FUN 15 Positioning TO17_VEH_02 In-Vehicle Equipment |  |
| TO17 FUN 16 Application TO17_ALG_02 Demonstration of Algorithm Process |  |
| TO17 FUN 17 Signal Control TO17_INF_01 Infrastructure Readiness |  |
| TO17 FUN 18 DSRC Communication (Infrastructure) TO17_INF_01 Infrastructure Readiness |  |
| TO17 FUN 19 In-Vehicle Processing TO17_VEH_02 In-Vehicle Equipment |  |
| TO17 FUN 20 Vehicle Trajectory Planning Algorithm TO17_ALG_01 Demonstration of Algorithm Functionality |  |
| TO17 FUN 21 Vehicle CANBus Data | REMOVED |
| TO17 FUN 22 DSRC Message Data TO17_DATA_01 Output Data Post-Processing |  |

Task Category ID Description Test Case # Test Case Name

TO17 FUN 23 Vehicle Positioning System TO17_DATA_01 Output Data Post-Processing

TO17 FUN 24 Vehicle Longitudinal Control TO17_DATA_01 Output Data Post-Processing

TO17 FUN 25 MAP Parser TO17_DATA_01 Output Data Post-Processing

TO17 FUN 26 SPaT Parser TO17_DATA_01 Output Data Post-Processing

TO17 FUN 27 MAP Matching TO17_ALG_02 Demonstration of Algorithm Process

TO17 BEH 1 Ignition to Park TO17_VEH_04 Vehicle Behavior

TO17 BEH 2 Park to Manual TO17_VEH_04 Vehicle Behavior

TO17 BEH 3 Park to Automated TO17_VEH_04 Vehicle Behavior

TO17 BEH 4 Automated to Manual TO17_VEH_04 Vehicle Behavior

TO17 BEH 5 Manual to Automated TO17_VEH_04 Vehicle Behavior

TO17 BEH 6 Automated to EcoDrive TO17_VEH_04 Vehicle Behavior

TO17 BEH 7 EcoDrive to Automated TO17_VEH_04 Vehicle Behavior

TO17 BEH 8 EcoDrive to Idle TO17_VEH_04 Vehicle Behavior

TO17 BEH 9 Idle to Automated TO17_VEH_04 Vehicle Behavior

TO17 BEH 10 Idle to Manual TO17_VEH_04 Vehicle Behavior

TO17 INT 1 Standard Wired Electrical Interface TO17_VEH_02 In-Vehicle Equipment

TO17 INT 2 Standard Command Structure TO17_VEH_01 Documentation of Vehicle Functionality

TO17 SEC 1 Physical Security TO17_SEC_01 Prototype Security

TO17 SEC 2 Operational Security TO17_SEC_01 Prototype Security

TO17 SEC 3 System Access TO17_SEC_02 PC Password Protection

TO17 DVI 1 System Activation Switch TO17_DVI_01 Driver-Vehicle Interface

TO17 DVI 2 System Status Indicator TO17_DVI_01 Driver-Vehicle Interface

TO17 DVI 3 Intersection Departure Switch TO17_DVI_01 Driver-Vehicle Interface

TO17 DVI 4 Signal Phase and Timing Display TO17_DVI_01 Driver-Vehicle Interface

TO17 DVI 5 Motion Status Indicator TO17_DVI_01 Driver-Vehicle Interface

TO17 DVI 6 Auditory Indicator TO17_DVI_01 Driver-Vehicle Interface

TO17 DVI 7 Auditory Alert TO17_DVI_01 Driver-Vehicle Interface

TO17 DAT 1 Data Logging Elements TO17_DATA_01 Output Data Post-Processing

Task Category ID Description Test Case # Test Case Name

TO17 DAT 2 Data Logging Frequency TO17_DATA_01 Output Data Post-Processing

TO17 DAT 3 Data Logging Time Protocol TO17_DATA_01 Output Data Post-Processing

TO17 DAT 4 Data Storage TO17_DATA_01 Output Data Post-Processing

# Appendix A - GlidePath Security Protocol Agreement

GLIDEPATH SECURITY PROTOCOL AGREEMENT

This document outlines the required protocols to ensure secure and safe operation of the GlidePath Prototype Vehicle and Application. The signatories below document their agreement to abide by these protocols and to ensure that any new project staff members are trained accordingly.

I.	Safety

The driver and all passengers SHALL wear safety belts when the vehicle is in motion.

II.	System Access

The prototype platform (Linux PC) SHALL be protected by a single-layer authentication with a username and password to be shared only with project staff.

III.	Physical Security

The prototype ignition and door key SHALL be stored in secured, locked location when not in use to avoid improper use by unauthorized personnel.

IV.	Operational Security

The prototype SHALL NOT be left unattended while the vehicle is running to avoid improper use by unauthorized personnel.

The signatures below indicate the staff members who have been approved by the FHWA General Task Manager to operate the GlidePath Application. Any new users must be approved by the FHWA General Task Manager, trained by an approved operator, and must sign an addendum to this agreement to be stored on file with the FHWA General Task Manager.

Khalid Ahmad	Date

Julie Evans	Date

Christopher Armstrong	Date

John Stark	Date

Dave Ference	Date

Kyle Rush	Date

I, Osman Altan, authorize the project staff listed above to operate the GlidePath Prototype

Vehicle and Application.

FHWA GTM Signature	Date

# Appendix B - Acronyms

| ASD | Aftermarket Safety Device |
| --- | --- |
| DSRC | Dedicated Short Range Communications |
| DVI | Driver-Vehicle Interface |
| GTM | General Task Manager |
| JAUS | Joint Architecture for Unmanned Systems |
| MPD | Motion Profile Driver |
| SPaT | Signal Phase and Timing |
| WSA | WAVE (Wireless Access for Vehicular Environments) Service Advertisement |